Attribute Description for Shape Files based on Model Results

IJ_GIS: I_J index in the GIS grid shapefile.

I_MOD: I index used in the model, and I_MOD = I_GIS +1.

J_MOD: J index used in the model, and J_MOD = J_GIS +1.

BED_CHG: cumulative bed elevation change (cm).

BED_MAP: bed map values, -1  hard bottom, 0 cohesive bed, 1  non-cohesive bed.

SCO:  maximum scour depth occurred so far (cm).

FRAC10_1A: fraction of class-1A sediment at the top-10-cm layer.

FRAC10_1B: fraction of class-1B sediment at the top-10-cm layer.

FRAC10_2: fraction of class-2 sediment at the top-10-cm layer.

FRAC10_3: fraction of class-3 sediment at the top-10-cm layer.

Some shape files contains followings:

F10_BED1A (1B, C2, and C3): bed source contents of class-1A (1B, 2, and 3) sediment at the top-10-cm layer (fraction).

F_BED = (F10_BED1A + F10_BED1B + F10_BEDC2 + F10_BEDC3) * 100, i.e., bed source content at the top-10-cm layer (%).

F10_US1A (1B, C2, and C3): upstream source contents of class-1A (1B, 2, and 3) sediment at the top-10-cm layer (fraction).

F_UP = (F10_US1A + F10_US1B + F10_USC2 + F10_USC3) * 100, i.e., upstream source content at the top-10-cm layer (%).

F10_LA1A (1B, C2, and C3): lateral source contents of class-1A (1B, 2, and 3) sediment at the top-10-cm layer (fraction).

F_LAT = (F10_LA1A + F_LAT1B+ F_LATC2+ F_LATC3) *100, i.e., lateral source content at the top-10-cm layer (%).

R = (F_BED – 100)/number of years

SEDRATE: cumulative sedimentation rate (cm/yr).

HALFTIME:  half-time of bed source content at the top-10-cm layer  [year].